AGRICULTURAL ROBOT
DESIGN & PERFORMANCE SUMMARY

| Category | Parameter | Summary |
| --- | --- | --- |
| Performance | Standard operating speed | ~6 km/h target for low-speed field operation |
| Battery | Battery capacity | 12 V, 18 Ah = 216 Wh |
| Battery | Estimated runtime | ~3–4 hours continuous operation |
| Battery | Estimated travel distance | ~18–24 km on one-time charge at ~6 km/h, before field-turning/terrain losses |
| Solar | Solar panel | 20 W; supplements the battery and extends endurance under suitable sunlight |
| Robot | Estimated total mass | ~5.8–6.2 kg |
| Robot | Target operating/load requirement | Approximately 10–12 kg, subject to testing |
| Chassis | Chassis size | 12 × 16 in, 1 mm metal sheet |
| Drive | Drive system | 4-wheel differential drive; one motor per wheel |
| Motors | Motor type | 4 × JGB37-520, 12 V geared DC motors |
| Wheels | Wheel diameter | 125 mm |
| Control | Controller | ESP32 DevKit |
| Control | Motor speed control | ESP32 PWM |
| Control | Direction control | H-bridge input logic |
| Control | IMU role | Motion/orientation feedback for heading and path correction |
| Power | Battery | 12 V, 18 Ah (18,000 mAh); nominal 216 Wh |
| Power | Solar | 20 W supplemental solar panel |
| Power | Protection | Suitable charge controller, battery protection, fuse protection, and correctly sized wiring |
| Charging | Theoretical solar charging time | 10.8 hours from 216 Wh ÷ 20 W; practical charging takes longer due to losses and variable sunlight |
| Field | 1 acre | 4,046.9 m² |
| Field | Effective coverage width | 12 in = 0.3048 m |
| Field | Approx. square field side | 63.6 m |
| Field | Approx. field passes | ~209 passes |
| Field | Approx. 180° end turns | ~208 turns |
| Field | Coverage note | Actual turns depend on field shape, headland space, overlap, and working width |

1. Project Purpose

What is the robot's purpose?

A compact autonomous platform for controlled movement over soil and predefined paths.

What is the chassis size?

12 × 16 in, made from 1 mm metal sheet.

Why four-wheel drive?

It improves traction, load distribution, stability, and reduces wheel slip on soil.

Which motors are used?

Four JGB37-520 12 V geared DC motors, one per wheel.

Why geared DC motors?

They provide higher wheel torque and controllable low-speed operation.

Why 125 mm wheels?

They balance traction, ground clearance, torque demand, and chassis size.

Which motor driver is preferred?

L298N is preferred for the prototype due to simplicity, availability, and cost.

2. Control & Electronics

How is motor speed controlled?

The ESP32 uses PWM to regulate motor speed.

How is direction controlled?

H-bridge input logic controls forward and reverse rotation.

What is the ESP32's role?

It reads sensors, executes control logic, and generates motor commands.

What is the IMU's role?

It provides motion/orientation feedback for heading and path correction.

How does the robot recover from a path deviation?

Differential wheel speeds steer the robot back toward the desired path.

What is the control architecture?

Sensors → ESP32 → motor drivers → four motors/wheels, with feedback.

3. Power System

What battery is used?

12 V, 18 Ah with nominal energy of 216 Wh.

Why is a 20 W solar panel used?

It supplements the battery and extends endurance under suitable sunlight.

What protection is required?

Use a suitable charge controller, battery protection, fuse protection, and correctly sized wiring.

How long is theoretical solar charging?

10.8 hours from 216 Wh ÷ 20 W; practical charging takes longer due to losses and variable sunlight.

4. Battery Runtime & Distance

What is the estimated runtime?

~3–4 h under the document's estimated average-load field conditions.

What is the average power example?

~55–70 W.

What is the standard operating speed?

~6 km/h.

How far can the robot travel on one-time charge?

**~18–24 km on one-time charge** at ~6 km/h, as a theoretical travel estimate.

Will actual field distance be lower?

Yes. Actual field coverage can be lower because of turns, stops, soil resistance, wheel slip, and terrain.

5. 1-Acre Coverage & Turns

What area is 1 acre?

4,046.9 m².

What effective coverage width is assumed?

12 in = 0.3048 m.

What is the approximate square-field side?

63.6 m.

How many field passes are estimated?

~209 passes.

How many 180° end turns are estimated?

~208 turns.

What affects the actual number of turns?

Field shape, headland space, overlap, and working width.

6. Mechanical Load

What is the estimated robot mass?

~5.8–6.2 kg, including chassis, motors, wheels, battery, electronics, solar panel, and hardware.

What load range is targeted?

The drivetrain is intended for a total operating/load requirement of approximately 10–12 kg, subject to testing.

Why is low mass important?

Lower mass reduces rolling resistance, traction demand, wheel slip, and motor current.

Component Specifications

| Component | Specification |
| --- | --- |
| Chassis | 12 × 16 in, 1 mm metal sheet; chassis mass ~1.9 kg |
| Drive | 4-wheel differential drive; one motor per wheel |
| Motors | 4 × JGB37-520, 12 V geared DC motors |
| Wheels | 125 mm diameter |
| Controller | ESP32 DevKit |
| Battery | 12 V, 18 Ah (18,000 mAh); nominal 216 Wh |
| Solar | 20 W solar panel; supplemental charging |
| IMU | Motion and orientation feedback |
| Motor Driver | L298N for prototype-level control |
| Target Robot Mass | ~5.8–6.2 kg |