AGRICULTURAL ROBOT
DESIGN & PERFORMANCE SUMMARY

| Category | Parameter | Summary |
| --- | --- | --- |
| Performance | Operating speed | ~5–6 km/h target for low-speed field operation |
| Battery | Capacity | 24 V, 100 Ah = 2,400 Wh nominal energy |
| Battery | Estimated runtime | ~5 h at assumed average load of ~480 W: t = E/P = 2400/480 = 5 h |
| Battery | Estimated travel distance | ~25–30 km theoretical at ~5–6 km/h before field losses |
| Solar | Solar panel | 200 W supplemental panel to extend endurance in sunlight |
| Robot | Total mass | ~19–20 kg design target including battery/chassis/solar hardware |
| Robot | Operating/load requirement | Designed around approximately 20 kg total operating mass |
| Chassis | Size | 24 × 36 in, 1 mm metal sheet |
| Drive | System | 4-wheel differential drive; one motor per wheel |
| Motors | Type | 4 × 24 V, 280 W geared DC motors |
| Motors | Total rated power | P_total = 4 × 280 = 1,120 W |
| Wheels | Diameter | 12 inch |
| Control | Controller | ESP32 DevKit |
| Control | Speed control | ESP32 PWM through motor drivers |
| Control | Direction | H-bridge motor-driver logic |
| Power | Battery | 24 V, 100 Ah rechargeable battery; E = V × Ah = 2,400 Wh |
| Power | Solar | 200 W supplemental solar panel |
| Power | Motor drivers | 1 × BTS7960 + 1 × Cytron MDD10A |
| Power | Protection | 24 V charge controller, BMS, fuses, emergency isolation and correctly sized wiring |
| Charging | Theoretical solar charging | t = 2,400 Wh ÷ 200 W ≈ 12 h; practical time is longer due to losses/sunlight |
| Field | 1 acre | 4,046.9 m² |
| Field | Effective coverage width | 24 in = 0.6096 m |
| Field | Approx. square-field side | √4,046.9 ≈ 63.6 m |
| Field | Approx. field passes | 4,046.9 ÷ (63.6 × 0.6096) ≈ 104.4 ≈ 105 passes |
| Field | Approx. 180° end turns | ~104 turns for ~105 passes |

1. Project Purpose

Q: What is the robot's purpose? 
 A compact autonomous platform for controlled movement over soil and predefined agricultural paths.

Q: What is the chassis size?  
 24 × 36 in using 1 mm metal sheet.

Q: Why four-wheel drive?  
 It improves traction, stability, load distribution and reduces wheel slip.

Q: Which motors are used?  
 Four 24 V, 280 W geared DC motors, one per wheel.

Q: Why 280 W motors?  
 They provide higher available wheel power and torque for the ~20 kg platform.

Q: Why 12-inch wheels?  
 They provide useful ground clearance and better mobility on uneven soil.

2. Control & Electronics

Q: How is motor speed controlled?  
 ESP32 PWM regulates motor speed through the motor drivers.

Q: How is direction controlled?  
 Motor-driver input logic commands forward and reverse rotation.

Q: What is the ESP32's role?  
 It reads sensors, runs control logic and generates motor commands.

Q: How does the robot recover from path deviation?  
 Differential wheel speeds steer the robot back toward the desired path.

Q: What is the control architecture?  
 Sensors → ESP32 → motor drivers → four motors/wheels, with feedback.

3. Power System

Q: What battery is used?  
24 V, 100 Ah; nominal energy E = V × Ah = 24 × 100 = 2,400 Wh.

Q: Why a 100 Ah battery?  
At ~480 W average load, t = 2,400/480 ≈ 5 h nominal.

Q: Why is a 200 W solar panel used?  
 It supplements the battery and can extend endurance under suitable sunlight.

Q: What protection is required?  
 Use a 24 V charge controller, BMS, fuses, emergency isolation and correctly sized wiring.

Q: How long is theoretical solar charging?  
 t = 2,400/200 ≈ 12 h, while practical charging takes longer due to losses and sunlight.

4. Battery Runtime & Distance

Q: What is the total motor rated power?  
P_total = 4 × 280 W = 1,120 W maximum rated motor power.

Q: What average power is assumed for ~5-hour runtime?  
 Approximately 400–480 W average system load.

Q: What is the estimated runtime?  
 At 480 W, t = 2,400 Wh ÷ 480 W ≈ 5 h nominal.

Q: Can the motors run continuously at 1,120 W for 5 hours?  
 2,400/1,120 ≈ 2.14 h before losses, so 5 h requires lower average power.

Q: What is the standard operating speed?  
~5–6 km/h target.

Q: How far can the robot travel on one charge?  
 ~25–30 km theoretical at ~5–6 km/h before field losses.

Q: Will actual field distance be lower?  
Yes, turns, stops, soil resistance, slip, payload and terrain reduce distance.

5. 1-Acre Coverage & Turns

Q: What area is 1 acre?  
A = 4,046.9 m².

Q: What effective coverage width is assumed?  
W = 24 in = 0.6096 m.

Q: What is the approximate square-field side?  
L = √A = √4,046.9 ≈ 63.6 m.

Q: How many field passes are estimated?  
N ≈ A/(L×W) = 4,046.9/(63.6×0.6096) ≈ 105 passes.

Q: How many 180° end turns are estimated?  
Approximately N − 1 ≈ 104 turns.

Q: What affects the actual number of turns?  
Field shape, headland space, overlap, working width and turning strategy.

6. Mechanical Load

Q: What is the revised robot mass?  
~19–20 kg total design target including battery, chassis and solar hardware.

Q: Why target ~20 kg?  
It provides a realistic compact agricultural-platform mass while retaining manageable traction and motor demand.

Q: Why is low mass important?  
 Lower mass reduces rolling resistance, slip, traction demand and motor current.

Q: What is the approximate weight force at 20 kg?  
F = mg = 20 × 9.81 ≈ 196 N.

Q: How does the load distribute over four wheels? 
 Ideal static load ≈ 196/4 ≈ 49 N per wheel.

7. Component Specifications

Q: What chassis is specified?  
 24 × 36 in, 1 mm metal sheet.

Q: What drive system is specified?  
 4-wheel differential drive with one motor per wheel.

Q: What motors are specified?  
 4 × 24 V, 280 W geared DC motors; total rated power 1,120 W.

Q: What wheels are specified?  
 12-inch diameter wheels.

Q: What controller is specified?  
ESP32 DevKit.

Q: What battery is specified?  
 24 V, 100 Ah; nominal energy 2,400 Wh.

Q: What solar panel is specified?  
200 W supplemental solar panel.

Q: What motor drivers are specified?  
 BTS7960 and Cytron MDD10A; verify exact motor-current compatibility.

Q: What protection is specified?  
 Charge controller, BMS, fuses, emergency isolation and correctly sized wiring.

8. Important Design Assumptions

Q: Is ~5-hour runtime guaranteed?  
No; it is an engineering estimate based on 2,400 Wh battery energy and ~400–480 W average load.

Q: What must be verified experimentally?  
 Actual motor current, battery usable capacity, runtime, traction and performance must be measured in field tests.

Q: Does 20 kg change the design target?  
 Yes; chassis, wheel traction, motor current and power budget should be checked for the ~20 kg total mass.