4-WHEEL ROBOT
MOTOR & LOAD ANALYSIS

Engineering Design Summary

1. Design Requirements

| Parameter | Specification / Recorded Value |
| --- | --- |
| Robot type | 4-wheel robot / four-wheel drive |
| Chassis / metal sheet | 16 × 12 inch |
| Metal sheet weight | 1.9 kg |
| Wheel diameter | 125 mm (12.5 cm) |
| Wheel radius | 62.5 mm (6.25 cm) |
| Number of wheels | 4 |

2. Geared DC Motor Specification

| Parameter | Specification |
| --- | --- |
| Motor model | GA25-370 geared DC motor |
| Rated voltage | 12 V DC |
| Speed | 280 RPM |
| Motor torque | 2 kg·cm (as recorded) |
| Motor quantity | 4 |
| Drive arrangement | One motor per wheel |

3. Tractive / Pushing Force Calculation

The approximate wheel force is calculated from: F = T / r

| Calculation | Result |
| --- | --- |
| Torque per motor | 2 kg·cm |
| Wheel radius | 6.25 cm |
| Force per motor | 2 / 6.25 = 0.32 kgf ≈ 3.14 N |
| Total force (4 motors) | 0.32 × 4 = 1.28 kgf ≈ 12.6 N |
| Approx. total pushing force | ≈ 1.28 kgf (≈ 12.6 N) |

4. Load / Feasibility Assessment

| Item | Assessment |
| --- | --- |
| Metal sheet mass | 1.9 kg |
| Four-wheel configuration | Suitable for distributing the load across four contact points |
| Motor system | 4 × GA25-370, 12 V, 280 RPM |
| Basic feasibility | The recorded design is feasible for a 4-wheel robot, subject to actual motor torque, gearing, battery current capability, and surface friction. |