# Agriculture Survey Rover

Phase 1 — Wi-Fi Controlled Forward / Backward Operation

MCU: ESP32-WROOM-32  |  Motor Driver: BTS7960 (43A H-Bridge)  |  Battery: 12V  |  Actuators: 12V DC Gear Motors

START

↓

Main power switch closed (12V supply to ESP32 + BTS7960)

↓

System Initialization Boot ESP32 (WROOM-32) ¦ BTS7960 driver set to safe/disabled ¦ Start Wi-Fi Access Point + status LED

↓

DECISION: 12V battery voltage OK AND client connected to ESP32 AP?

NO  →  Error Indicator (LED / buzzer alert)  →  HALT

↓  YES

Standby / Idle State Motors at rest, awaiting command

↓

Read Wi-Fi Control Input HTTP/WebSocket command from ESP32 AP — loop start

↓

DECISION: Wi-Fi link valid? (failsafe check)

NO  →  Failsafe: Stop Motors (output forced to zero)  →  loops back to "Read Wi-Fi Control Input"

↓  YES

Decode Command + Deadband Filter Map stick position to direction/speed; center wiggle treated as zero

↓

DECISION: Command type?

| FORWARD Set Forward Direction + scaled PWM speed | NEUTRAL Speed = 0 — Coast / no drive | BACKWARD Set Reverse Direction + scaled PWM speed |
| --- | --- | --- |

↓  (all three converge)

Actuate Motor Driver (BTS7960) R_EN/L_EN + RPWM/LPWM set per direction

↓

Movement Executing 12V DC gear motors drive rover forward / backward / hold — continuous loop

↻  loops back to "Read Wi-Fi Control Input"

## Legend

Green box — Start / Halt state

Blue box — Process / action

Orange box — Decision

Red text — Fault / failsafe path

Phase 1 of multi-phase rover control architecture — ESP32 + BTS7960, Wi-Fi forward/backward only.